Integrated Enterprise Network and IP Telephony System

Design and Simulation of a Four-Department Enterprise Network using Cisco Packet Tracer

Technologies & Concepts

VLAN • DHCP • OSPF • VoIP • Cisco CME • IP Telephony

Project By

Subhash Srija

Computer Networks | 2026

# Table of Contents

1. Introduction

2. Problem Statement

3. Objectives

4. Scope of the Project

5. System Requirements

6. Network Topology and Architecture

7. Department and Device Distribution

8. IP Addressing Scheme

9. Physical Connections

10. VLAN Design

11. Router-on-a-Stick Configuration

12. DHCP Configuration

13. OSPF Routing

14. IP Telephony and CME

15. Dial-Peer Configuration

16. Network and Telephony Operation

17. Testing and Verification

18. Final Results

19. Advantages

20. Limitations

21. Future Scope

22. Conclusion

23. Frequently Asked Questions and Doubts

# 1. Introduction

This project presents the design and simulation of a small enterprise communication network using Cisco Packet Tracer. The organization is divided into four departments: Administration, Human Resources (HR), Finance, and Sales. Each department has its own data and voice networks while the departments remain interconnected through a four-router WAN ring.

The network combines several core Computer Networks concepts: VLAN-based segmentation, router-on-a-stick inter-VLAN communication, DHCP, OSPF dynamic routing, IP telephony, Cisco CallManager Express (CME), and VoIP dial peers. The design provides separate logical networks for ordinary data traffic and voice traffic and enables users in different departments to communicate through IP phones.

Cisco 2811 routers are used as departmental gateways and for WAN routing. WIC-2T serial modules provide the required serial WAN interfaces. Cisco 2960 switches connect departmental end devices. IP phones also provide pass-through connectivity for laptops where applicable.

# 2. Problem Statement

A conventional departmental network may require separate physical infrastructure for voice and data and may rely on static routing or poorly segmented LANs. This project addresses the need for a structured enterprise network in which departmental traffic is logically separated, addresses are assigned automatically, routers exchange routes dynamically, and IP phones can communicate across departments.

The objective is to simulate an enterprise environment that is scalable, organized, and suitable for demonstrating practical networking concepts in Cisco Packet Tracer.

# 3. Objectives

Design a four-department enterprise network in Cisco Packet Tracer.

Create separate data and voice VLANs for each department.

Provide inter-VLAN connectivity using router-on-a-stick.

Use DHCP to automatically assign IP addresses to data devices and IP phones.

Use OSPF for dynamic routing between the four departmental routers.

Configure Cisco CME for IP-phone registration and extension management.

Configure VoIP dial peers for inter-department telephone calls.

Test end-to-end IP connectivity and inter-department voice communication.

# 4. Scope of the Project

The project is a simulated enterprise network rather than a production deployment. It focuses on LAN segmentation, WAN connectivity, routing, address assignment, and VoIP communication.

# 5. System Requirements

| Component | Quantity / Specification |
| --- | --- |
| Simulation software | Cisco Packet Tracer |
| Routers | 4 × Cisco 2811 |
| Serial modules | WIC-2T on each router |
| Switches | 4 × Cisco 2960 |
| IP phones | 19 |
| Laptops | 22 |
| Printers | 4 |
| Routing protocol | OSPF |
| Voice technology | Cisco CME / SCCP and VoIP |
| Address assignment | DHCP |
| LAN design | Data VLAN + Voice VLAN per department |

# 6. Network Topology and Architecture

The routers form a ring topology. Each router represents one department and connects to its departmental switch. The ring provides two WAN neighbors for each router.

WAN connections:

| Link | Network | Endpoint A | Endpoint B |
| --- | --- | --- | --- |
| R1–R3 | 10.0.0.0/30 | R1 S0/3/0 = 10.0.0.1 | R3 S0/3/0 = 10.0.0.2 |
| R1–R2 | 10.0.0.4/30 | R1 S0/3/1 = 10.0.0.5 | R2 S0/3/0 = 10.0.0.6 |
| R3–R4 | 10.0.0.8/30 | R3 S0/3/1 = 10.0.0.9 | R4 S0/3/0 = 10.0.0.10 |
| R2–R4 | 10.0.0.12/30 | R2 S0/3/1 = 10.0.0.13 | R4 S0/3/1 = 10.0.0.14 |

# 7. Department and Device Distribution

| Department | Laptops | IP Phones | Printer | Total |
| --- | --- | --- | --- | --- |
| Administration | 6 | 4 | 1 | 11 |
| HR | 5 | 6 | 1 | 12 |
| Finance | 7 | 3 | 1 | 11 |
| Sales | 4 | 6 | 1 | 11 |
| Total | 22 | 19 | 4 | 45 |

In departments where laptops are connected through IP phones, the phone provides a pass-through connection. This reduces the number of switch ports and physical cables required. The switch port is configured with both an access data VLAN and a voice VLAN, allowing the phone and attached laptop to use separate logical networks.

# 8. IP Addressing Scheme

| Department | Data VLAN | Data Gateway | Voice VLAN | Voice Gateway |
| --- | --- | --- | --- | --- |
| Administration | 10 – 192.168.10.0/24 | 192.168.10.1 | 110 – 192.168.110.0/24 | 192.168.110.1 |
| HR | 20 – 192.168.20.0/24 | 192.168.20.1 | 120 – 192.168.120.0/24 | 192.168.120.1 |
| Finance | 30 – 192.168.30.0/24 | 192.168.30.1 | 130 – 192.168.130.0/24 | 192.168.130.1 |
| Sales | 40 – 192.168.40.0/24 | 192.168.40.1 | 140 – 192.168.140.0/24 | 192.168.140.1 |

## 8.1 Router Interface Addressing

| Router | Interface | IP Address | Purpose |
| --- | --- | --- | --- |
| R1_Admin | Fa0/0.10 | 192.168.10.1/24 | Administration Data |
| R1_Admin | Fa0/0.110 | 192.168.110.1/24 | Administration Voice |
| R1_Admin | S0/3/0 | 10.0.0.1/30 | R1–R3 WAN |
| R1_Admin | S0/3/1 | 10.0.0.5/30 | R1–R2 WAN |
| R2_HR | Fa0/0.20 | 192.168.20.1/24 | HR Data |
| R2_HR | Fa0/0.120 | 192.168.120.1/24 | HR Voice |
| R2_HR | S0/3/0 | 10.0.0.6/30 | R2–R1 WAN |
| R2_HR | S0/3/1 | 10.0.0.13/30 | R2–R4 WAN |
| R3_Finance | Fa0/0.30 | 192.168.30.1/24 | Finance Data |
| R3_Finance | Fa0/0.130 | 192.168.130.1/24 | Finance Voice |
| R3_Finance | S0/3/0 | 10.0.0.2/30 | R3–R1 WAN |
| R3_Finance | S0/3/1 | 10.0.0.9/30 | R3–R4 WAN |
| R4_Sales | Fa0/0.40 | 192.168.40.1/24 | Sales Data |
| R4_Sales | Fa0/0.140 | 192.168.140.1/24 | Sales Voice |
| R4_Sales | S0/3/0 | 10.0.0.10/30 | R4–R3 WAN |
| R4_Sales | S0/3/1 | 10.0.0.14/30 | R4–R2 WAN |

# 9. Physical Connections

Each departmental router uses FastEthernet0/0 to connect to FastEthernet0/1 of its departmental 2960 switch.

| Department | Router | Router Port | Switch Port |
| --- | --- | --- | --- |
| Administration | R1_Admin | Fa0/0 | Fa0/1 |
| HR | R2_HR | Fa0/0 | Fa0/1 |
| Finance | R3_Finance | Fa0/0 | Fa0/1 |
| Sales | R4_Sales | Fa0/0 | Fa0/1 |

The four routers are connected using serial links through their WIC-2T modules. Serial DCE/DTE links are used for the WAN ring.

## 9.1 Departmental Switch Port Layout

Administration:

- Fa0/2–Fa0/5 connect to the four IP phones

- Fa0/6–Fa0/7 connect to laptops

- Fa0/8 connects to the printer

- Phones with attached laptops use the phone's PC/pass-through port.

HR:

- Fa0/2–Fa0/7 connect to six IP phones

- Fa0/8 connects to the printer

- laptops are attached through the relevant IP phones.

Finance:

- Fa0/2–Fa0/4 connect to three IP phones with laptops through the phones

- Fa0/5–Fa0/8 connect to four additional laptops

- Fa0/9 connects to the printer.

Sales:

- Fa0/2–Fa0/5 connect to four IP phones with laptops through the phones

- Fa0/6–Fa0/7 connect to two additional phone-only connections

- Fa0/8 connects to the printer.

# 10. VLAN Design

| Department | Data VLAN | Voice VLAN | Names |
| --- | --- | --- | --- |
| Administration | 10 | 110 | ADMIN_DATA / ADMIN_VOICE |
| HR | 20 | 120 | HR_DATA / HR_VOICE |
| Finance | 30 | 130 | FINANCE_DATA / FINANCE_VOICE |
| Sales | 40 | 140 | SALES_DATA / SALES_VOICE |

The router-to-switch link is configured as an 802.1Q trunk carrying both the department's data and voice VLANs. End-device ports that connect an IP phone and laptop use the data VLAN as the access VLAN and the voice VLAN as the voice VLAN.

# 11. Router-on-a-Stick Configuration

Router-on-a-stick allows one physical router interface to serve multiple VLANs using subinterfaces. Each subinterface has an IP address that acts as the default gateway for its VLAN.

Example – R1_Admin

interface Fa0/0.10
 encapsulation dot1Q 10
 ip address 192.168.10.1 255.255.255.0

interface Fa0/0.110
 encapsulation dot1Q 110
 ip address 192.168.110.1 255.255.255.0

The same design is applied to the other departments with VLAN IDs 20/120, 30/130, and 40/140.

# 12. DHCP Configuration

Each router acts as the DHCP server for its department. Separate DHCP pools are used for data and voice. The first ten addresses are excluded so that gateway/reserved addresses are not dynamically assigned. For voice networks, DHCP option 150 points phones to the CME/TFTP source address.

## 12.1 HR DHCP

ip dhcp excluded-address 192.168.20.1 192.168.20.10
ip dhcp pool HR_DATA
 network 192.168.20.0 255.255.255.0
 default-router 192.168.20.1
 dns-server 8.8.8.8

ip dhcp excluded-address 192.168.120.1 192.168.120.10
ip dhcp pool HR_VOICE
 network 192.168.120.0 255.255.255.0
 default-router 192.168.120.1
 option 150 ip 192.168.120.1

## 12.2 Finance DHCP

ip dhcp excluded-address 192.168.30.1 192.168.30.10
ip dhcp pool FINANCE_DATA
 network 192.168.30.0 255.255.255.0
 default-router 192.168.30.1
 dns-server 8.8.8.8

ip dhcp excluded-address 192.168.130.1 192.168.130.10
ip dhcp pool FINANCE_VOICE
 network 192.168.130.0 255.255.255.0
 default-router 192.168.130.1
 option 150 ip 192.168.130.1

## 12.3 Sales DHCP

ip dhcp excluded-address 192.168.40.1 192.168.40.10
ip dhcp pool SALES_DATA
 network 192.168.40.0 255.255.255.0
 default-router 192.168.40.1
 dns-server 8.8.8.8

ip dhcp excluded-address 192.168.140.1 192.168.140.10
ip dhcp pool SALES_VOICE
 network 192.168.140.0 255.255.255.0
 default-router 192.168.140.1
 option 150 ip 192.168.140.1

The Administration DHCP configuration follows the same pattern using 192.168.10.0/24 and 192.168.110.0/24, with 192.168.10.1 and 192.168.110.1 as gateways.

# 13. OSPF Routing

OSPF process 1 is used in area 0. Each router advertises its two departmental LAN networks and its two WAN /30 networks. OSPF dynamically learns routes to remote departments.

| Router | OSPF Router ID | Advertised Networks |
| --- | --- | --- |
| R1_Admin | 1.1.1.1 | 192.168.10.0/24, 192.168.110.0/24, 10.0.0.0/30, 10.0.0.4/30 |
| R2_HR | 2.2.2.2 | 192.168.20.0/24, 192.168.120.0/24, 10.0.0.4/30, 10.0.0.12/30 |
| R3_Finance | 3.3.3.3 | 192.168.30.0/24, 192.168.130.0/24, 10.0.0.0/30, 10.0.0.8/30 |
| R4_Sales | 4.4.4.4 | 192.168.40.0/24, 192.168.140.0/24, 10.0.0.8/30, 10.0.0.12/30 |

Example – R4_Sales

router ospf 1
 network 192.168.40.0 0.0.0.255 area 0
 network 192.168.140.0 0.0.0.255 area 0
 network 10.0.0.8 0.0.0.3 area 0
 network 10.0.0.12 0.0.0.3 area 0

OSPF neighbor verification showed FULL adjacency between the routers. R4, for example, formed FULL adjacencies with R3 (3.3.3.3) and R2 (2.2.2.2).

# 14. IP Telephony and CME

Cisco CallManager Express functionality is configured on each departmental router. Each router acts as the call-processing endpoint for its local voice VLAN. IP phones receive their voice IP address through DHCP and use option 150 to locate the CME/TFTP source.

| Department | CME Source Address | Max Ephones | Max DNs | Extensions |
| --- | --- | --- | --- | --- |
| Administration | 192.168.110.1 | 4 | 4 | 1101–1104 |
| HR | 192.168.120.1 | 6 | 6 | 1201–1206 |
| Finance | 192.168.130.1 | 3 | 3 | 1301–1303 |
| Sales | 192.168.140.1 | 6 | 6 | 1401–1406 |

Example – Sales CME

telephony-service
 max-ephones 6
 max-dn 6
 ip source-address 192.168.140.1 port 2000
 auto assign 1 to 6

Phones registered successfully in the working simulation. The physical phone number does not necessarily correspond to the CME ephone number because auto-registration can assign ephone/DN entries according to registration order. The important requirement is that each phone receives a valid extension and registers successfully.

# 15. Dial-Peer Configuration

Dial peers define how calls destined for another department are routed over the WAN. The destination pattern identifies the extension range and the session target identifies the next router used to reach that department.

## 15.1 Finance Dial Peers

dial-peer voice 4 voip
 destination-pattern 11..
 session target ipv4:10.0.0.1

dial-peer voice 8 voip
 destination-pattern 12..
 session target ipv4:10.0.0.6

dial-peer voice 11 voip
 destination-pattern 14..
 session target ipv4:10.0.0.10

## 15.2 Sales Dial Peers

dial-peer voice 6 voip
 destination-pattern 11..
 session target ipv4:10.0.0.5

dial-peer voice 10 voip
 destination-pattern 12..
 session target ipv4:10.0.0.13

dial-peer voice 12 voip
 destination-pattern 13..
 session target ipv4:10.0.0.9

The final working Packet Tracer simulation was tested with inter-department calls. During troubleshooting, the Sales-to-Administration path required particular attention because the call initially failed even though IP reachability and other VoIP paths were working. After the configuration/state settled, the call became operational without changing the physical topology.

# 16. Network and Telephony Operation

A data device first obtains an address from the departmental DHCP pool. The IP phone obtains an address from the voice DHCP pool. The switch separates the two traffic classes using the access VLAN and voice VLAN. The router subinterfaces provide the departmental gateways.

When a user calls an extension in another department, the local CME uses the appropriate VoIP dial peer. The call is forwarded toward the remote router through the OSPF-routed WAN. The remote CME handles the destination extension and rings the corresponding IP phone.

OSPF is responsible for IP reachability; CME and dial peers are responsible for the VoIP call path. These are separate functions that work together to provide end-to-end communication.

# 17. Testing and Verification

The completed simulation was verified using Packet Tracer connectivity tests and router/switch verification commands.

| Test | Expected Result | Observed Result |
| --- | --- | --- |
| DHCP – data devices | Automatic data IP address | Successful |
| DHCP – IP phones | Automatic voice IP address | Successful |
| Phone registration | Phones show REGISTERED | Successful |
| OSPF adjacency | Neighbors reach FULL state | Successful |
| Router-to-router ping | WAN endpoints reachable | Successful |
| R4 to Finance gateway | 192.168.130.1 reachable | Successful |
| R4 to Finance phone | 192.168.130.12 reachable | Successful |
| Admin ↔ HR call | Call connects | Successful |
| Admin → Finance call | Call connects | Successful |
| Admin → Sales call | Call connects | Successful |
| HR ↔ Finance call | Call connects | Successful |
| HR → Sales call | Call connects | Successful |
| Finance ↔ Sales call | Call connects | Successful |
| Sales → Admin call | Initially failed during troubleshooting; later worked | Successful in final working state |

## 17.1 Useful Verification Commands

show ip route
show ip ospf neighbor
show ephone
show running-config | section telephony-service
show running-config | section dial-peer
show running-config | include destination-pattern

These commands were used because they are supported by the Packet Tracer router model used in the project.

# 18. Final Results

The final simulation successfully demonstrates a four-department enterprise network with separate data and voice VLANs, automatic DHCP addressing, OSPF-based dynamic routing, registered IP phones, and inter-department VoIP calling.

The network successfully provided communication between Administration, HR, Finance, and Sales while maintaining separate logical voice and data networks within each department.

# 19. Advantages

Logical separation of voice and data traffic using VLANs.

Automatic IP assignment through DHCP.

Dynamic routing using OSPF rather than maintaining static routes.

Centralized departmental call processing through CME.

Inter-department VoIP communication without requiring separate telephone wiring.

Four-router ring topology provides multiple WAN paths.

IP-phone pass-through can reduce switch-port and cabling requirements.

Design can be expanded by adding users, phones, VLANs, or departments.

# 20. Limitations

The project is a Packet Tracer simulation and does not represent all behavior of physical Cisco equipment.

Real-world voice quality mechanisms such as advanced QoS are not implemented.

Security controls such as AAA, port security, ACL policies, and encrypted management are not comprehensively configured.

The simulated CME/VoIP environment is limited by Packet Tracer's supported features.

No connection to an actual PSTN or external SIP provider is included.

# 21. Future Scope

Implement QoS to prioritize voice traffic.

Add ACLs and switch security features.

Introduce redundant routers/switches and gateway redundancy.

Add centralized network monitoring using SNMP/syslog.

Integrate enterprise authentication and role-based access.

Connect the VoIP system to a real SIP/PSTN service in a physical or more advanced virtual lab.

Expand the topology with additional branches and departments.

# 22. Conclusion

This project successfully demonstrates the design and simulation of an enterprise IP network integrating data communication and IP telephony. The four departments are logically separated using data and voice VLANs, while router-on-a-stick provides gateway services. DHCP automates address assignment, OSPF dynamically distributes routes across the WAN ring, and Cisco CME provides local IP-phone registration and extension management.

The completed network demonstrates end-to-end connectivity and inter-department telephone communication. The project therefore provides practical implementation of several important Computer Networks concepts in a single integrated Cisco Packet Tracer environment.

# 23. Frequently Asked Questions and Project Doubts

This section collects the important questions and doubts raised during the project build and explains the underlying networking concepts.

## 23.1 Why did we add the WIC-2T module to the Cisco 2811 routers?

The Cisco 2811 has limited built-in FastEthernet interfaces, while the project requires a LAN connection plus two WAN serial connections. The WIC-2T module adds serial interfaces so the routers can form the four-router WAN ring. Serial DCE/DTE connections are used for the router-to-router links.

## 23.2 Why are laptops connected through IP phones?

An IP phone commonly has a PC/pass-through port. A laptop can therefore connect through the phone while the switch port carries both the data VLAN and the voice VLAN. This reduces the number of switch ports and physical cables required and demonstrates voice/data VLAN separation.

## 23.3 What is the purpose of Option 150 in DHCP?

DHCP Option 150 tells Cisco IP phones the address of the TFTP server used to obtain their configuration/firmware information. In this project, the CME router's voice gateway address is used as the Option 150 address.

## 23.4 Why do we use separate data and voice VLANs?

Separating voice and data creates logical traffic isolation. Data devices use the departmental data VLAN, while IP phones use the voice VLAN. This makes the network easier to organize and provides a foundation for applying voice-specific policies such as QoS.

## 23.5 Why do we use router-on-a-stick?

A single physical router interface can serve multiple VLANs by using 802.1Q subinterfaces. Each subinterface is assigned to one VLAN and acts as that VLAN's default gateway. This avoids requiring a separate physical router interface for every VLAN.

## 23.6 Why do we use OSPF?

OSPF dynamically exchanges routing information between routers. Instead of manually configuring routes to every departmental network, each router advertises its directly connected networks and learns remote networks through OSPF.

## 23.7 Why is the topology a ring?

The ring connects every router to two neighboring routers. It provides an alternate direction through the WAN for reaching another part of the network and demonstrates dynamic routing in a multi-router environment.

## 23.8 What is CME?

Cisco CallManager Express (CME) provides call-processing functions on a Cisco router. In this project it manages IP-phone registrations, directory numbers, and local call handling.

## 23.9 What is an ephone?

An ephone is a CME configuration object representing an IP phone. CME uses ephone entries to associate a phone with its MAC address and directory number.

## 23.10 What is an ephone-dn?

An ephone-dn represents a directory number/extension in CME. Examples in this project include 1101–1104 for Administration and 1401–1406 for Sales.

## 23.11 What is a dial peer?

A dial peer tells the router how to handle a particular telephone number pattern. For example, a destination pattern such as 11.. represents four-digit Administration extensions beginning with 11.

## 23.12 What is the difference between OSPF and dial peers?

OSPF provides IP routing between routers and makes remote networks reachable. Dial peers determine how VoIP calls are forwarded toward another CME/VoIP endpoint. OSPF and dial peers therefore solve different parts of the communication problem.

# Appendix A – Final Configuration Summary

| Router | Data GW | Voice GW | WAN Interfaces |
| --- | --- | --- | --- |
| R1_Admin | 192.168.10.1 | 192.168.110.1 | S0/3/0 10.0.0.1; S0/3/1 10.0.0.5 |
| R2_HR | 192.168.20.1 | 192.168.120.1 | S0/3/0 10.0.0.6; S0/3/1 10.0.0.13 |
| R3_Finance | 192.168.30.1 | 192.168.130.1 | S0/3/0 10.0.0.2; S0/3/1 10.0.0.9 |
| R4_Sales | 192.168.40.1 | 192.168.140.1 | S0/3/0 10.0.0.10; S0/3/1 10.0.0.14 |

# Appendix B – Final Call Matrix

| Source | Destination | Final Status |
| --- | --- | --- |
| Administration | HR | Working |
| Administration | Finance | Working |
| Administration | Sales | Working |
| HR | Administration | Working |
| HR | Finance | Working |
| HR | Sales | Working |
| Finance | Administration | Working |
| Finance | HR | Working |
| Finance | Sales | Working |
| Sales | Administration | Working |
| Sales | HR | Working |
| Sales | Finance | Working |

All documented call paths were verified as working in the final project state.

End of documentation